Content

- Scope

- Definitions and abbreviations

- Personnel pre-qualifications

3.1 Medical fitness

3.2 Education and training

- Procedure

4.1 Principle

4.2 Samples

4.3 Equipment and materials

4.4 Reagents and solutions

4.5 Detailed instructions

4.6 Reporting

4.7 Quality control

4.8 Waste management

- Related documents

Annex 1.  How to hand wash

Annex 2. Incident report form

|  | Compiled by | Examined by | Approved by | Replaced | New version |
| --- | --- | --- | --- | --- | --- |
| Name |  |  |  | Code: | Code: |
| Date |  |  |  |  |  |
| Signature |  |  |  |  |  |
| Laboratory area: |  | No of copies: | Reason for change: |  |  |

- Scope

This SOP describes hygiene and safety procedures for worker protection against laboratory-acquired tuberculosis in the event of an accidental spill of specimens in an acid-fast bacilli microscopy laboratory.

- Definitions and abbreviations

spill of specimens

Accidental spillage of specimens before they have been subjected to appropriate waste management.

accident

An undesired event giving rise to death, ill-health, injury, damage, loss or distress.

incident

An event that gives rise to an accident or has the potential to lead to an accident.

good microbiological technique (GMT)

Working methods designed to eliminate or minimize exposure to pathogens via, for example, aerosols, splashes, ingestion, absorption, accidental inoculation.

AFB:	acid fast bacilli

NA:	not applicable

- Personnel pre-qualifications

- Medical fitness

In accordance with national laws and practices, arrangements should be made for  appropriate health surveillance of TB laboratory workers:

- before enrolment in the TB laboratory;

- at regular intervals thereafter, annually or bi-annually;

- after any biohazard incident;

- at the onset of TB symptoms.

Ideally, individual medical records shall be kept for up to 10 years following the end of occupational exposure.

Laboratory workers should be educated about the symptoms of TB and provided with ready access to free medical care if symptoms arise.

Confidential HIV counselling and testing should be offered to laboratory workers. Options for reassignment of HIV-positive or immuno-suppressed individuals away from the high-risk areas of the TB laboratory should be considered.

All cases of disease or death identified in accordance with national laws and/or practice as resulting from occupational exposure to biological agents shall be notified to the competent authority.

- Education and training

Education and training must be given on the following topics:

- potential risks to health (symptoms of TB disease and transmission);

- precautions to be taken to minimize aerosol formation and prevent exposure;

- hygiene requirements;

- wearing and use of protective equipment and clothing;

- handling of potentially infectious materials;

- prevention of incidents and steps to be taken by workers in the case of incidents (biohazard incidents, chemical, electrical and fire hazards);

- laboratory design, including airflow conditions;

- good laboratory practice;

- waste handling;

- organization of work flow.

The training shall be:

- given before a staff member takes up his/her post

- strictly supervised;

- adapted to take account of new or changed conditions; and

- repeated periodically, preferably every year.

- Procedure

4.1 Principle

Accidental spills of specimens should be cleaned up immediately.

4.2 Samples

All specimens handled in an AFB microscopy laboratory

4.3 Equipment and materials

Absorbent tissue

Thick rubber gloves

Forceps

Waste containers (bucket, plastic bags)

4.4 Reagents and solutions

Disinfectants (and procedures for their use) according to national guidelines (or see SOP on the use of disinfectants).

4.5 Detailed instructions

- Place absorbent tissue over the spill area and apply disinfectant solution liberally.

- Leave covered for a minimum of 2 hours.

- Pick up the contaminated material using forceps and thick rubber gloves for protection against cuts from glass debris, if any, and place in the waste container.

- Clean surfaces with a final wash of disinfectant

- If damaged, rubber gloves are discarded; if not, they can be disinfected.

- Wash hands after the clean-up is complete (see Annex 1).

4.6 Reporting

Notify the head of the laboratory of the incident.

Every incident/accident must be documented and records must be kept in the laboratory archives (Annex 2). All corrective action must be similarly documented.

4.7 Quality control

NA

4.8 Waste management

All materials used in the clean-up should be treated as infectious waste.

Disposable materials should be autoclaved or incinerated.

If a burning drum is used, collect contaminated material (containers with tightened caps, sticks, etc…) from the laboratory in a bucket containing phenol 5%. Burn contents weekly. When cool, bury the residue at a depth of at least 1 metre.

Reusable materials (e.g. forceps, transport trays) can be disinfected using appropriate disinfectants (e.g. 5% phenol) before washing.

5. Related documents

Biosafety in microbiological and biomedical laboratories, 5th ed. Washington, DC, 2007. United States Department of Health and Human Services/Centers for Disease Control and Prevention/National Institutes of Health, 2007.

Laboratory biosafety manual, 3rd ed. Geneva, World Health Organization, 2004.

Rieder HL et al. The public health service national tuberculosis reference laboratory and the national laboratory network. Minimum requirements, role and operation in a low-income country. Paris, International Union Against Tuberculosis and Lung Disease, 1998.

Standards Australia/Standards New Zealand. Safety in laboratories - microbiological aspects and containment facilities. Sydney, Standards Australia International, 2002.

Annex 1.  How to hand wash (http://www.who.int/gpsc/tools/HAND_WASHING.pdf)

Annex 2. Incident report form

| Institution: |  |
| --- | --- |
| Laboratory designation: |  |
| Head of the laboratory: |  |
| Date, time of the incident: |  |
| Nature of the incident (what happened?) |  |
| Extent of the incident: - estimate of the volume of contaminated spill - details of the volume of broken glass (if any) - description of the surfaces contaminated - list of material/equipment contaminated (if any) - list of samples involved in the incident |  |
| List of persons present in the laboratory when the incident occurred |  |
| List of persons with physical injury (if any) Nature of physical injury |  |
| Name of the physician in charge of the first medical aid |  |
| Name of the physician in charge of the medical record and further medical surveillance |  |
| Corrective action: - how to prevent such an incident; - how to improves staff adherence to safety procedures |  |